RED CELL SOLUTIONS

Physical Security  |  Social Engineering  |  Security Training

PHYSICAL ACCESS CONTROL ASSESSMENT

Locks  •  Access Control Systems  •  Perimeter Security

Comprehensive evaluation of all physical entry points, access control mechanisms, and perimeter defenses to identify unauthorized access vulnerabilities.

| STANDARD LEAD TIME 2–3 Weeks | INVESTMENT $50,000 |
| --- | --- |
| TOTAL CHECKPOINTS 127 Items | ASSESSMENT CATEGORIES 12 Sections |

INTERNAL DOCUMENT  —  ASSESSMENT CHECKLIST

1. PHYSICAL LOCKS

☐  Lock type identification (deadbolt, mortise, padlock, cam, etc.)

☐  Lock grade/rating (ANSI/BHMA Grade 1, 2, or 3)

☐  Manufacturer and model documentation

☐  Key control policy review (who has keys, how are they tracked)

☐  Key duplication resistance (restricted keyway, patent protection)

☐  Master key system analysis (if applicable)

☐  Bypass vulnerability testing (picking, bumping, shimming, impressioning)

☐  Bypass time documentation per lock

☐  Physical condition and wear assessment

☐  Photo documentation of each lock and deficiency found

Notes: _________________________________________________________________________

2. ELECTRONIC / SMART LOCKS

☐  Lock type and manufacturer identification

☐  Authentication method review (PIN, biometric, RFID, Bluetooth, multi-factor)

☐  Default credential check (factory PINs, unchanged admin passwords)

☐  Brute force resistance testing

☐  Man-in-the-middle vulnerability assessment

☐  Replay attack susceptibility

☐  Tamper alarm functionality verification

☐  Fail-safe vs. fail-secure configuration review

☐  Battery/power failure behavior testing

☐  Firmware version and patch status

☐  Audit log review (completeness, retention period, tamper evidence)

☐  Remote management security (if cloud-connected)

Notes: _________________________________________________________________________

3. DOORS AND ENTRY POINTS

☐  Door material and construction assessment (solid core, hollow core, metal, glass)

☐  Door frame integrity check

☐  Handle/hardware type documentation

☐  Door gap measurement (top, bottom, latch side, hinge side)

☐  Latch slipping / credit card bypass testing

☐  Hinge exposure assessment (removable pins, exterior-facing hinges)

☐  Strike plate security (screws, reinforcement)

☐  Door sensor presence and functionality

☐  Automatic closer functionality and hold-open compliance

☐  Emergency exit hardware (panic bars, alarmed exits) verification

☐  Propping detection capability

☐  Photo documentation per door

Notes: _________________________________________________________________________

4. BADGE / CREDENTIAL SYSTEMS

☐  Card technology identification (125kHz prox, 13.56MHz smart, iCLASS, DESFire, SEOS)

☐  Encryption level assessment

☐  Clone-ability / duplication testing

☐  Credential format and facility code review

☐  Expired / revoked credential audit

☐  Lost badge reporting and deactivation process review

☐  Credential issuance and lifecycle management policy

☐  Multi-factor authentication implementation (badge + PIN, badge + biometric)

☐  Visitor badge differentiation and visual identification

☐  Audit log review (access granted, denied, time stamps)

☐  Anti-passback configuration check

☐  Credential database hygiene (orphaned accounts, former employees)

Notes: _________________________________________________________________________

5. ACCESS CONTROL SYSTEM (HEAD-END)

☐  System manufacturer, model, and software version

☐  Server/controller physical security (location, who has access)

☐  Network segmentation review (dedicated VLAN / air-gapped)

☐  Admin account audit (number of admins, password policy, shared accounts)

☐  Role-based access control configuration review

☐  Integration points inventory (fire alarm, elevator, intercom, CCTV)

☐  Alarm and event monitoring workflow (who sees alerts, response time)

☐  System redundancy and failover capability

☐  Backup and disaster recovery procedures

☐  Patch management and update cadence

☐  Audit log integrity and retention compliance

Notes: _________________________________________________________________________

6. PERIMETER SECURITY

☐  Full perimeter walk and mapping

☐  Fence line assessment (height, material, condition, climb resistance)

☐  Gate functionality and locking mechanism review

☐  Barrier identification (bollards, planters, jersey barriers, vehicle barriers)

☐  Barrier bypass method assessment

☐  Vehicle entry control points (arms, gates, tire shredders)

☐  Pedestrian entry control points

☐  Blind spot identification and mapping

☐  Lighting assessment (coverage, lux levels, dark zones)

☐  Signage evaluation (warning signs, trespassing notices, wayfinding)

☐  Natural surveillance lines of sight

☐  Adjacent property and public access risk assessment

Notes: _________________________________________________________________________

7. CAMERAS / CCTV / VIDEO SURVEILLANCE

☐  Camera inventory (type, location, manufacturer, model)

☐  Coverage map vs. blind spot analysis

☐  Camera functionality verification (live feed, no dead cameras)

☐  Resolution and image quality assessment (day and night)

☐  Night vision / IR capability testing

☐  Field of view and angle review per camera

☐  Recording system type (NVR, DVR, cloud)

☐  Data retention policy and actual storage duration

☐  Monitoring protocol (live monitoring vs. review-only)

☐  Alert and motion-triggered recording configuration

☐  Tampering detection (camera masking, repositioning, signal loss alerts)

☐  Remote access security for viewing/management

☐  Physical security of recording equipment

Notes: _________________________________________________________________________

8. TAILGATING / PIGGYBACKING

☐  Observed tailgating attempt testing at each controlled entry

☐  Mantrap / airlock / vestibule presence and functionality

☐  Turnstile or optical barrier presence and effectiveness

☐  Employee awareness and challenge culture assessment

☐  Anti-tailgating technology evaluation (sensors, weight plates, optical counters)

☐  Signage and policy communication at entry points

☐  Social engineering resistance testing (following someone through with a delivery)

Notes: _________________________________________________________________________

9. CPTED (CRIME PREVENTION THROUGH ENVIRONMENTAL DESIGN)

☐  Natural surveillance assessment (sight lines, window placement, open layouts)

☐  Natural access control evaluation (landscaping, pathways guiding foot traffic)

☐  Territorial reinforcement review (signage, fencing, maintained grounds signaling ownership)

☐  Maintenance and image assessment (broken window theory indicators)

☐  Lighting design review for crime deterrence

☐  Landscaping audit (overgrown areas providing concealment)

☐  Entrapment spot identification (alcoves, recessed doorways, stairwells)

☐  Activity support analysis (does the environment encourage legitimate use)

Notes: _________________________________________________________________________

10. VISITOR MANAGEMENT

☐  Visitor registration process review (pre-registration, walk-in, digital vs. paper)

☐  Visitor identity verification method (ID check, photo capture)

☐  Visitor badge issuance and visual differentiation

☐  Escort policy and compliance observation

☐  Visitor access scope (where can they go, what is restricted)

☐  Visitor credential expiration and auto-deactivation

☐  Visitor log completeness and data retention

☐  Credential sharing / pass-along testing

☐  After-hours visitor protocol

☐  Contractor and vendor management procedures

☐  Delivery and loading dock access procedures

Notes: _________________________________________________________________________

11. ALARM SYSTEMS

☐  Alarm panel type, manufacturer, and monitoring service

☐  Motion detector coverage and functionality testing

☐  Glass break sensor placement and functionality testing

☐  Door/window contact sensor inventory and testing

☐  Duress / panic alarm availability and testing

☐  Signal jamming / communication interference vulnerability

☐  Cellular backup for alarm communication

☐  Response time documentation (alarm to notification to response)

☐  False alarm rate and nuisance alarm management

☐  Arm/disarm procedures and code management

☐  Zone configuration review

☐  Alarm log and history review

Notes: _________________________________________________________________________

12. DELIVERABLES

☐  Physical Security Gap Report with photo evidence per finding

☐  Vulnerability severity scoring (Critical / High / Medium / Low)

☐  Prioritized remediation roadmap with recommended sequencing

☐  Cost estimates per recommended upgrade

☐  Executive summary formatted for leadership / board presentation

☐  Optional: findings briefing / walkthrough session with stakeholders

Notes: _________________________________________________________________________

RED CELL SOLUTIONS

redcellsolutions.com

CONFIDENTIAL — Prepared exclusively for prospective clients of Red Cell Solutions.